УДК 621.378.525 : 535.8

V. M. Zemljanskiy,  doctor of physical and mathematical sciences,A.P.Chydesov,E.V.Tomala

Research spatial cohesion of laser radiation LDMS.

The principle of action onboard laser Doppler’s measurer of speed and corners of attack (LDMS) is based on measurement of three orthogonal components of a vector of air speed with the subsequent processing of this information by the specialized processor. In such LDMS the powerful laser in polymodal a regime is used, spatial cohesion of which radiation signal / noise and angular divergence of laser bunches essentially influences the attitude.The error of measurement of the module of a degree spatial cohesion by known ways and devices makes up to 10... 15 %, that is not enough at metrology of certification LDMS. The practical circuit of measurement of the module of a degree spatial cohesion is developed, in which are eliminated as tool errors, which are conditioned by change of a spatial rule interference of a picture because of instability interferometer, and methodical error of measurement because of change of a corner between laser bunches. Is shown, that at the expense of indemnification of these errors the accuracy of measurement raises more than on the order.

УДК 621.378.525

## В.М.Землянський, доктор фіз.-мат.наук, А.П.Чудесов,О.В.Томаля

### Дослідження просторової когерентності лазерного випромінювання ЛДВШ

Принцип дії бортового лазерного доплерівського вимірювача швидкості і кутів атаки (ЛДВШ) грунтується на вимірюванні трьох ортогональних компонент вектора повітряної швидкості з наступною обробкою цієї інформації спеціалізованим процесором. В таких ЛДВШ використовується потужний лазер в багатомодовому режимі, просторова когерентність випромінювання якого суттєво впливає на відношення сигнал/шум і кутове розходження лазерних пучків.Похибка вимірювання модуля степені просторової когерентності відомими способами і пристроями складає до 10…15%, що недостатньо при метрологічній атестації ЛДВШ. Розроблена практична схема вимірювання модуля степені просторової когерентності, в якій усунені як інструментальні похибки, які обумовлені зміненням просторового положення інтерференційної картини через нестабільність інтерферометра, так і методична похибка вимірювання обумовлена зміненням кута між лазерними пучками. Показано, що за рахунок компенсації цих похибок точність вимірювання підвищується більше ніж на порядок.