### P. Bezoušek, PhD, University of Pardubice, Czech Republic

V. Kubeček,, PhD , ERA Inc., Pardubice, Czech Republic

## accuracy Analysis of A 3D mixed TOA/DOA passive surveillance system VERA

Abstract.

The passive surveillance system VERA serves as a backup air traffic control aid. It consists of three or more terrestrial stations receiving simultaneously signal of an airborne SSR transponder. The emitter position  is then usually evaluated using the corresponding Times Of Arrivals (TOA) measurement. In this paper the Direction Of Arrival (DOA) measurement addition to a standard 3D system  is analysed and its effect on the aircraft  position accuracy is evaluated.

- The 3D VERA system description.

Fig.1. Basic configuration of a 3D

four position passive system.

d

d

R

L

Q

C

d



x

The system under consideration consists of three side receiving stations R, L, Q, a central receiving station C and of a central processing station PS deployed ideally in a triangle according to the Fig.1. The signal, received in the lateral receiving station is retransmitted by a microwave link  to the central processing station where the TOA’s of the signals are measured and aircraft position is calculated. Sophisticated SW ensures  an automatic tracking of each  aircraft  and generates an airspace picture and  a standard radar report for remote ATC centers. The system is designed to process the SSR transponder replies in A/C and S modes at frequency 1090 MHz or DME interrogation signals.

PS

Central. process. station

Signal Conditioning

Measur. Unit

Reply Receiver

Micr. Link Transmitter

SIDE receiving station - l

Reply Receiver

Micr. Link receiver

central receiving station -c

Video

TTL

UPS

Communicat.

& Control

FM  Modul.

Diagn.

BP Filter

BP Filter

LNA

MIX

IF

LOG

Detectoror

Pulse

Selector

GPS

receiver

Command

Situation Display

Diagnostics

Display

Output Report

PC - II

PC - I

UPS

Target Positioning &Tracking

Micr. Link receiver

Micr. Link receiver

LO

Target Track &Tracking

Fig. 2. Block diagram of the passive surveillance system.

The block diagram of the system is shown in the Fig. 2.

A selection of  received pulses on the basis of their pulsewidth  is performed after detection. A brodband microwave link is used to transmit the signals to the central station in real time. The modulation  of the transmitted signal is performed by  mixing the stable local oscillator with another VCO modulated 2-FSK.

At the central station the signals are fed to the measurement unit – a specialised card in the computer PC-I where their TOA´s are measured and respective messages are decoded. Then they are correlated and position of the emitter is evaluated.  The GPS receiver delivers the time reference for the whole system. The second computer PC-II generates output reports and data for auxiliary situation display and serves for communication and maintenance.

2. Technical characteristics

The system:

coverage			Rmax = 350 km, azimuth 360, elevation - full zenith

capacity				300 automatically tracked targets

output data			X, Y, altitude, A, C, 1, 2, S address, track quality, update rate 5 sec

The receiving station:

The microwave link:

Frequency band			13 GHz

Modulation			FSK, f=8 MHz

Antenna			 = 60 cm,

G =35,5 dB

Maximum range		70 km

Receiver sensitivity 		-95 dBm

at S/N = 25 dB

Noise figure		5 dB

Bandwidth (IF)		14 MHz

Dynamic range		70 dB

IF			70 MHz, logarithmic

Output pulses		TTL,

0,45  0,15 s/3dB,

leading 70 ns, trailing 120 ns

- Position accuracy calculation.

Position calculation is based on a set of equations of the following form:

t0+Ti+Ti0+τi=tiarctgy0−yix0−xi=ϕ i i=1, 2, 3, 4arctgz0−ziri=ϑ i ri=x0−xi2+y0−yi2					(1)

TOA measurements

DOA measurements

where: t0	....  the time of the reply at the aircraft

Ti	....  the propagation time between aircraft and the i-th receiving station

Ti0	....  the propagation time between the i-th receiving station and the central station

i	....  the processing time-delay at the i-th station

ti	....  the TOA of the signal primarily received at the i-th station measured at the

central station

x0, y0	....  horizontal coordinates of the aircraft

xi, yi	....  horizontal coordinates of the i-th station

z0, zi	…. vertical coordinates of the aircraft and the i-th station

i	....  relative azimuth of the aircraft, measured at the i-th station.

i	....  relative elevation of the aircraft, measured at the i-th station.

A standard linearization procedure leads to a set of linear equations for aircraft local coordinates u in the vicinity of the selected point r0 depending on deviations of measured quantities Q  (ti, i, i) at presence  of various errors in TOA and DOA measurement, represented generally by an error vector W:	Cr0 u+W=ΔQ								(2)

where: C(r0)	…. matrix of derivatives of the left-hand side of  (1) respective to the aircraft coordinates.

The equation (2) has been studied elsewhere (1, 2). In 2 we discussed the effects of long term variations and systematic errors  on the system accuracy. Recently a complex system using mixed TOA and DOA measurement was suggested for aircraft  position calculation and signal separation 3.  In the present  paper we concentrate on short term fluctuations and particularly on possibility of 3D system measurement accuracy enhancement by adding direction of arrival measurements. The standard deviations of TOA measurement in the VERA system due to short term fluctuations is estimated equally to t = 10 ns in all channels (2). The standard deviations of  DOA measurement depend on antennas dimensions and S/N ratios.  Here we assume that all four stations produce equal standard deviations of  azimuth  and elevation   measurement.

25 m

50 m

v= 150

m

Fig.3. Vertical rms errors of a pure TOA based system.     d = 8 km, h = 2 km, t = 10 ns.

As an example of  computed results we may see in the Fig. 3 the vertical position accuracy in a constant altitude h = 2 km, with the central to side station distances d = 8 km with only TOA measurement. Comparable  results have been achieved during  the fly test, carried out at Pardubice under the management  of EUROCONTROL (Fig. 4). The aircraft altitude was measured with accuracy of 65 ft (RMS) along direct parts of  its trajectory.

The comparison of a real TOA-measurement-based system and a hypothetical mixed TOA/DOA-measurement system is shown at the Fig. 5.  We may see that adding  only azimuth measurement has a small effect on the observed parameters except of in the case of a very good angle-resolution antenna ( = 0.1). The same is true for both the horizontal and vertical  position accuracy. But adding elevation measurement substantially enhance the accuracy even in the case of a low resolution antenna ( = 1) which would be obtained with reasonably small antennas.  The plausible effect of elevation measurement on accuracy is more pronounced  than that of the azimuth measurement (namely at the longer range) obviously due to relatively low aperture in a vertical plane of the pure  TOA based system for low altitude targets.

4. Conclusions.

The passive system VERA is an application of a multisensor data fusion  processing and represents a principally new, high performance and relatively simple and low cost multifunction device. The system is produced at ERA Inc. Pardubice in several versions, designed for various applications as an ATC surveillance aid, an airport surface movement control tool, an independent height measuring equipment  or as a passive air defence system.  The same concept is a hot candidate as a ground based backup of an Automatic Dependent System - Broadcast after 2010 and is now intensively examined in USA and Europe as well.  Nevertheless further enhancement of the system accuracy is permanently actual. In this paper we examined the effect of adding the direction of arrival measurement in azimuth and elevation planes.  We find out that this could be particularly useful in the case of short based systems and preferably when  DOA measurement in the elevation plane is performed.

References

[1]  Koenke E. J., Geyer M., Lay R., Vilcans J.: Beacon Multilateration System to Facilitate ADS-B, The 53rd ION Conference Proceedings, Albuquerque, June 30 - July 2, 1997

2  Bezoušek P., Kubeček V., Štěrba P.: A Passive Radar Surveillance System VERA for ATC, Proc. of the IRS99 Conference, Vol I, Munich 1998, pp. 39 – 48

3  Petrochilos N., van Genderen P.: A New Approach to Handle the Secondary Surveillance Radar Replies, Int. Conference on Radars, Brest, France, May 1999

Fig. 4. A fly test at Pardubice, base distance of  the passive system d = 8 km. The measured altitude profile in comparison with  the mode C replies record (the smooth curve) is in the lower window.

Pardubice

Test aircraft trajectory

1

1

2

2

3

3

4

4

5

5

6

6

7

7

Fig. 5. Position errors in mixed TOA/DOA measurement system. d = 2km, h = 2 km, azimuth cut  =30. 1…. TOA only t = 10 ns , 2 …. TOA+ azimuth  = 1, 3 …. TOA+ azimuth  = 0.5, 4 …. TOA+ azimuth  = 0.1, 5 …. TOA + elev.  = 1, 6 …. TOA+ elev.  = 0.5, 7 …. TOA+ elev.  = 0.1